| V 2.8 HL7 Proposal |  |
| --- | --- |
| Change Request ID: | To be Defined |
| File Name: | EAC_Additional_Segment_DST.doc |
| Description: | Introduce an additional Destination segment in the EAC message. |
| Status: | New Proposal |
| Sponsoring Person | Andrzej J. Knafel |
| Sponsoring Business Unit | Roche Diagnostics Ltd. |
| Date Originated: | #### Replacement for the withdrawn change proposal 663 sent to OO Wiki (Hans) on 12/02/2010 |
| Date HL7 approved: |  |
| Backward Compatible: | #### Yes |
| Forward Compatible: | #### Yes |
| HL7 Status & Date |  |

# Justification Detail:

To support the possibility to express the destinations and routes in the transport and aliquot commands, an additional new segment should be introduced in the EAC message. The new DST segment should be used use for TT (Transport To) and AF (Aliquot From container), AT(Aliquot To container) commands.

# Open Issues:

Not known.

# Change Request Impact:

Not known.

# Documentation Changes:

The repeatability of this segment should be used for multiple destinations in sequence and the remark may be used as additional information, e.g. for destination dependent bar code labels for the secondary (aliquot) containers.

The repeatability of the Specimen Container block enables sending commands for Pooling specimen (multiple sources) from a Primary Specimen. Command for multiple aliquots requires specific Command group for each aliquot.

| Segments | Description | Status | Chapter |
| --- | --- | --- | --- |
| MSH | Message Header |  | 2 |
| [{SFT}] | Software Segment |  | 2 |
| [UAC] | User Authentication Credential |  | 2 |
| EQU | Equipment Detail |  | 13 |
| { | --- COMMAND begin |  |  |
| ECD | Equipment Command Detail |  | 13 |
| [ TQ1 ] | Timing/quantity |  | ? |
| { [ | --- SPECIMEN_CONTAINER begin |  |  |
| SAC | Specimen Container Detail |  | 13 |
| { [ SPM ] } | Specimen |  | 7 |
| { [ DST ] } | Transport destination |  | 13 |
| ] } | --- SPECIMEN_CONTAINER end |  |  |
| [ CNS ] | Clear Notification |  | 13 |
| } | --- COMMAND end |  |  |
| [ ROL ] | Role Detail |  | 12 |

### 13.4.13	DST – Transport Destination Segment

The Transport Destination segment contains information relevant for transport of the specimen container to specific destination on the specific equipment. This segment should be used in conjunction with the TT, AF, and AT commands of the ECD segment used in the EAC message.

HL7 Attribute Table –DST – Transport Destination

| SEQ | LEN | C.LEN | DT | OPT | RP/# | TBL# | ITEM # | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | CWE | R |  | ???? | ????? | Destination |
| 2 |  |  | CWE | O | Y | ???? | ????? | Route |

#### 13.4.13.0	DST Field Definitions

#### 13.4.13.1	DST-1  Destination (CWE)   ?????

Components:  <Identifier (ST)> ^ <Text (ST)> ^ <Name of Coding System (ID)> ^ <Alternate Identifier (ST)> ^ <Alternate Text (ST)> ^ <Name of Alternate Coding System (ID)> ^ <Coding System Version ID (ST)> ^ <Alternate Coding System Version ID (ST)> ^ <Original Text (ST)> ^ <Second Alternate Identifier (ST)> ^ <Second Alternate Text (ST)> ^ <Name of Second Alternate Coding System (ID)> ^ <Second Alternate Coding System Version ID (ST)> ^ <Coding System OID (ST)> ^ <Value Set OID (ST)> ^ <Value Set Version ID (DTM)> ^ <Alternate Coding System OID (ST)> ^ <Alternate Value Set OID (ST)> ^ <Alternate Value Set Version ID (DTM)> ^ <Second Alternate Coding System OID (ST)> ^ <Second Alternate Value Set OID (ST)> ^ <Second Alternate Value Set Version ID (DTM)>

Definition:  This field identifies the destination for transport of a specific container. This is a user defined table.

#### 13.4.13.2	DST-1  Route (CWE)   ?????

Components:  <Identifier (ST)> ^ <Text (ST)> ^ <Name of Coding System (ID)> ^ <Alternate Identifier (ST)> ^ <Alternate Text (ST)> ^ <Name of Alternate Coding System (ID)> ^ <Coding System Version ID (ST)> ^ <Alternate Coding System Version ID (ST)> ^ <Original Text (ST)> ^ <Second Alternate Identifier (ST)> ^ <Second Alternate Text (ST)> ^ <Name of Second Alternate Coding System (ID)> ^ <Second Alternate Coding System Version ID (ST)> ^ <Coding System OID (ST)> ^ <Value Set OID (ST)> ^ <Value Set Version ID (DTM)> ^ <Alternate Coding System OID (ST)> ^ <Alternate Value Set OID (ST)> ^ <Alternate Value Set Version ID (DTM)> ^ <Second Alternate Coding System OID (ST)> ^ <Second Alternate Value Set OID (ST)> ^ <Second Alternate Value Set Version ID (DTM)>

Definition:  This field identifies the route for transport of a specific container. This is a user defined table.

Repeatability of the Route means a composition of different segments into a route.